Lab task 01:

Diagram explanation:

- The 5 V source pushes current through the series resistors.

- Because they are in series, the same current flows through both R1 and R2.

- The voltage divides between them according to their resistance values:

  - R1 (larger resistance) gets a bigger share of voltage (≈3.03 V).

  - R2 (smaller resistance) gets less voltage (≈1.97 V).

- The sum of voltages across R1 and R2 equals the supply voltage (≈5 V), which confirms Kirchhoff’s Voltage Law.

Stimulated Values:

| R1 Voltage | R2 Voltage | Current |
| --- | --- | --- |
| 3.03 V | 1.97 V | 0.595 mA |

Solution:

1. Total Resistance

Rtotal=R1+R2=5100 Ω+3300 Ω=8400 Ω

2. Current in the Circuit

Using Ohm’s Law:

I=VRtotal=58400≈0.000595 A

So the current is:

I≈0.595 mA

3. Voltage Across R1

VR1=I⋅R1=0.000595⋅5100≈3.03 V

4. Voltage Across R2

VR2=I⋅R2=0.000595⋅3300≈1.97 V

---------------------------------------------------------------------------------------

Lab task 02:

Diagram Explanation:

- The 9 V source is applied across all three resistors simultaneously.

- Because of the parallel connection, each resistor experiences the full 9 V.

- The currents differ: smaller resistances allow more current to flow (R₃ has the largest current, R₁ the smallest).

- The total current is the sum of all branch currents (14.4 mA).

- The voltage between node 2 and node 3 is zero, since both are tied to the same potential in the parallel network.

Stimulated Values:

| R1 Current | R2 Current | R3 Current | Voltage (Node 2–3) |
| --- | --- | --- | --- |
| 0.9 mA | 4.5 mA | 9 mA | 0 V |

Solution:

1. Current through R₁

IR1=VR1=910000=0.0009 A

IR1=0.9 mA

2. Current through R₂

IR2=VR2=92000=0.0045 A

IR2=4.5 mA

3. Current through R₃

IR3=VR3=91000=0.009 A

IR3=9 mA

4. Total Current Supplied

Itotal=IR1+IR2+IR3=0.9+4.5+9=14.4 mA

5. Voltage Between Node 2 and Node 3

Since both nodes are directly connected to the same voltage source (9 V across all parallel branches), the voltage difference between node 2 and node 3 is:

V2-3=0 V

-------------------------------------------------------------------------------------

Post Lab explanation:

- The battery is the source of energy.

- R1 ensures controlled current flow.

- SW1 & SW2 act like gates to control charging/discharging.

- AM1 shows how much current is charging the capacitor.

- VM1 shows the capacitor’s voltage buildup.

- AM2 shows the current through the bulb.

- The bulb demonstrates the practical effect of current flow (light output).

-----------------------------------------------------------------------