STEPS for Inverting Amplifier:

Inverting Amplifier Steps (Proteus)

- Place OPAMP (Pick Devices) in Proteus

- Connect + terminal to ground

- Connect 1 kΩ resistor from input signal to – terminal

- Connect 3 kΩ resistor from output to – terminal (feedback)

6.

Give

+12

V supply

to op

-

amp

7.

Connect ground properly (common ground for input and circuit)

8.

Run simulation

Result

•

Output is

sine wave

•

Frequency =

40

Hz (same as input

)

•

Amplitude =

3

times input (≈ 6 V

)

•

Output is

inverted (180° phase shift)

STEPS

for

No

n

Inverting Amplifier:

Non

-

Inverting Amplifier (Proteus Steps)

1.

Place OPAMP (Pick Devices) in Proteus

2.

Connect + terminal to 2 V sine wave, 40 Hz input

3.

Connect 1 kΩ resistor from

–

terminal to ground

4.

Connect

2

kΩ resistor from output to

–

terminal

5.

Give +12 V supply to op

-

amp

6.

Connect proper ground for circuit

5. Apply 2 V sine wave, 40 Hz at input

7. Run simulation

Result

- Output is sine wave

- Frequency = 40 Hz

- Amplitude = 4 times input (≈ 8 V)

•

Output is same phase (not inverted)