Experiment 1:

Waveform Measurements in a Two-Resistor Series Circuit

Objective and Setup:

The primary goal of this experiment was to master the use of electronic test equipment, specifically the signal generator and oscilloscope, to measure fundamental electrical quantities in a basic resistive circuit. The setup required a breadboard, a power supply, connecting wires, and a multimeter.

Methodology:

The procedure began by constructing Circuit ‘A’, a series circuit consisting of two resistors:

R1 = 5.1kohm and R2 = 3.3kohm. Using the signal generator, a sinusoidal input signal (V1) was applied to the circuit with a frequency of 100 Hz and a peak-to-peak voltage Vpp of 5V.

Measurements:

To analyze the circuit behavior, the oscilloscope was used with a dual-probe configuration.

- Probe 1 was placed across the signal generator to monitor the input waveform constantly.

- Probe 2 was alternated between the terminals of R1 and R2 to measure the individual voltage drops.

For each component, the peak-to-peak voltage and time-related parameters (such as period) were measured from the oscilloscope display. These waveforms were then sketched onto the provided grids, and the RMS voltage and P-P voltage values were calculated and recorded in the data table.

Experiment 2:

Waveform Measurements in a Three-Resistor Series Circuit

Objective and Setup:

This experiment expanded on the initial task by increasing circuit complexity to observe voltage distribution across multiple components and combinations of resistors.

Methodology:

The procedure involved building Circuit ‘B’, which added a third resistor to the series chain. The components used were R1 = 4.7kohm, R2 = 3.3kohm and R3 = 5.1kohm. The signal generator parameters were adjusted to a higher frequency of 200 Hz and a peak-to-peak voltage of 7V.

Measurements and Analysis:

Measurement followed a systematic approach using the oscilloscope probes to capture a wider variety of data points:

- Individual Voltages: Probes were used to measure the voltage signal across R1, R2 and R3 individually.

- Combined Voltages: Measurements were also taken across pairs of resistors, specifically the total voltage drop across R1 and R2, as well as across R2 and R3.

For every configuration, the resulting sinusoidal waveforms were drawn on the oscilloscope grids. The final step involved filling out the comprehensive data table, which included the Input Voltage, RMS Voltage, and P-P Voltage for all individual and combined resistor segments to verify how the total source voltage was distributed across the series network.