Circuit Analysis (Lab # 6)

Experiment 6:  VERIFICATION OF KIRCHHOFFS LAWS

Circuit - KCL

Calculations:

Step 1: Node Relation

At the junction:

I1=I2+I3

Step 2: Equivalent Resistance

The parallel branch is:

Req=220⋅1000220+1000≈180.3 Ω

So total resistance:

Rtotal=330+Req≈330+180.3=510.3 Ω

Step 3: Current from Source

I1=ERtotal

- For E=5V:

I1=5510.3≈0.0098 A=9.8 mA

- For E=15V:

I1=15510.3≈0.0294 A=29.4 mA

- For E=25V:

I1=25510.3≈0.0490 A=49.0 mA

Step 4: Voltage Across Parallel Branch

Vbranch=I1⋅Req

- At 5 V:

Vbranch=9.8×10-3⋅180.3≈1.77 V

- At 15 V:

Vbranch=29.4×10-3⋅180.3≈5.31 V

- At 25 V:

Vbranch=49.0×10-3⋅180.3≈8.85 V

Step 5: Currents in Each Branch

I2=Vbranch220,I3=Vbranch1000

- At 5 V supply

I2=1.77220≈8.05 mA

I3=1.771000≈1.77 mA

Check:

I2+I3≈9.82 mA≈I1

- At 15 V supply

I2=5.31220≈24.1 mA

I3=5.311000≈5.31 mA

Check:

I2+I3≈29.4 mA≈I1

- At 25 V supply

I2=8.85220≈40.2 mA

I3=8.851000≈8.85 mA

Check:

I2+I3≈49.0 mA≈I1

---------------------------------------------------

Circuit - KVL

Calculations:

Step 1: Total Resistance

Rtotal=330+220+1000=1550 Ω

Step 2: Current in the Circuit

I=E1Rtotal

- For E1=5V:

I=51550≈0.00323 A=3.23 mA

- For E1=10V:

I=101550≈0.00645 A=6.45 mA

- For E1=15V:

I=151550≈0.00968 A=9.68 mA

Step 3: Voltage Drops Across Each Resistor

V=I⋅R

- At 5 V supply

V1=3.23×330≈1.06 V

V2=3.23×1000≈3.23 V

V3=3.23×220≈0.71 V

Check:

V1+V2+V3≈5.0 V

- At 10 V supply

V1=6.45×330≈2.13 V

V2=6.45×1000≈6.45 V

V3=6.45×220≈1.42 V

Check:

V1+V2+V3≈10.0 V

- At 15 V supply

V1=9.68×330≈3.19 V

V2=9.68×1000≈9.68 V

V3=9.68×220≈2.13 V

Check:

V1+V2+V3≈15.0 V

-----------------------------------------------------------